NOAA NESDIS

CENTER for SATELLITE APPLICATIONS and RESEARCH

GOES-R Advanced Baseline Imager (ABI) Routine Validation Tool Description Document For

Cloud and Moisture Imagery Product (CMIP)

Tim Schmit, NOAA/NESDIS/STAR

Kaba Bah, CIMSS

Mat Gunshor, CIMSS

Version 0.1

July 01, 2012

# TABLE OF CONTENTS

# TABLE OF CONTENTS	2

ACRONYMS	2

INTRODUCTION	3

1	INTRODUCTION	4

Purpose of This Document	4

Who Should Use This Document	4

Inside Each Section	5

2	PRODUCT OVERVIEW	5

2.1	Products Generated	6

2.2	Instrument Characteristics	6

VAKIDATION OVERVIEW	10

3.1	Algorithm Overview	10

ALGORITHM DESCRIPTION	10

GOES-R Simulated Input Data Sets	11

Select Links	25

GOES-R:	25

GOES:	25

UW/SSEC/CIMSS/ASPB:	25

REFERENCES	29

# LIST OF FIGURES

# LIST OF TABLES

# LIST OF ACRONYMS

ABI – Advanced Baseline Imager

AIT – Algorithm Integration Team

ARW – Advanced WRF

ATBD – Algorithm Theoretical Basis Document

AWG – Algorithm Working Group

AWIPS – Advanced Weather Interactive Processing System

BT – Brightness Temperature

BV – Brightness Value

CF – Climate and Forecasts

CFD – Continuous Full Disk

CIMSS – Cooperative Institute for Meteorological Satellite Studies

CMIP – Cloud and Moisture Imagery Product

CONUS – Continental United States

CRTM – Community Radiative Transfer Model

DC – Digital Count

FD – Full Disk

FGF – Fixed Grid Format

GFS – Global Forecast System

GPO – GOES-R Program Office

GRB – GOES ReBroadcast

GS – Ground Segment

GVAR – GOES Variable

HDF – Hierarchical Data Format

IDL – Interactive Data Language

IDV – Integrated Data Viewer

IR – InfraRed

ITT – ITT Industries

I&VT – Imagery and Visualization Team

IV&V – Independent Verification and Validation

KPP – Key Performance Parameter

McIDAS – Man computer Interactive Data Access System

NCSA – National Center for Supercomputing Applications

NetCDF – Network Common Data Format

PCI – [data visualization tool]

QC – Quality Control

SOI – Successive Order of Integration

SR – Scaled Radiance

TBD – To Be Determined

TOA – Top Of Atmosphere

TRR – Test Readiness Review

UTC – Universal Time Coordinated

UW – University of Wisconsin

WRF – Weather Research and Forecasting

# INTRODUCTION

The GOES-R ABI is designed to observe the Western hemisphere in various time intervals and at 0.5, 1, and 2 km spatial resolutions in visible, near-IR, and IR wavelengths. The ABI has two main scan modes, of which the most likely mode will allow a full disk image every 15 minutes, along with a Continental U.S. (CONUS) image every 5 minutes, and a mesoscale image as often as every 30 seconds. CMIP is the ABI tier 1A and Key Performance Parameter (KPP) product for GOES-R. There are 54 KPP Cloud and Moisture Imagery End-Products (CMIP).  This includes 16 ABI single-band images for 3 coverage areas plus the multi-band spectral products for each coverage area in NetCDF and McIDAS formats, which will be used as input data by various other algorithms.

Cloud and Moisture Imagery algorithms produce digital maps of clouds, moisture, and atmospheric windows, through which land and water are observed, from radiances for the visible, near-IR, and IR bands. Information will also be provided on the conversion from radiance to reflectance factor (ABI bands 1-6) and Brightness Temperatures (BT) (ABI bands 7-16). These will be converted into Brightness Values (BV), ranging from 0-4095 for a 12-bit band or 0-16383 for a 14-bit band, for display in a visualization system. Cloud and Moisture Imagery provides input to other algorithms producing other environmental products.

GOES-R ABI CMIP algorithms have been tested on ABI simulated data covering the CONUS and nearly Full Disk areas. The generated image files have been compared with the validation data generated by the research code, and the images are effectively identical.

# Information Volume relative to current GOES

Compared to current GOES, the ABI has improved spectral, spatial, and temporal image resolution by factors of 3, 4, and 5 respectively. In addition, there will be improved radiometric and image navigation and registration performance. This allows for not only improved and new uses directly with the imagery, but it also allows for improved and new products to be derived from the ABI data.

The ABI will improve every product from the current GOES Imager and will introduce a host of new products, such as cloud-top phase/particle size information and improved snow/ice detection, total column ozone, aerosol and smoke detection for air quality monitoring and forecasts. Other new products include vegetation monitoring and upper-level sulfur dioxide detection. The ABI will begin a new era in U.S. environmental remote sensing with more spectral bands, faster imaging, and higher spatial resolution than the current GOES imager.

# Purpose of the Imagery team

The purpose of the imagery team is generally two-fold:

(I) Demonstrate how to convert from GRB scaled radiances (e.g., GRB integers) to other physical units, such as radiance, brightness temperatures and brightness values.

(2) Build files that can be used for processing most of the ABI products, such as clouds, soundings, etc.

# Purpose of this document

This document provides a description for the routine validation tools for the Cloud and Moisture Imagery Product (CMIP) with simulated ABI images of the Advanced Baseline Imager (ABI) onboard the Geostationary Operational Environmental Satellite (GOES-R) series of NOAA geostationary meteorological/environmental satellites. The topics covered in this document include routine validation tools that are currently being developed or would be of great use for routine validation if developed, requirements for CMIP algorithms and software system, description of CMIP key algorithms, test data sets and outputs.

# Who should use this document

The intended users of this document include: algorithm reviewers, who would understand the theoretical basis of CMIP; product users, who would use the product in an operational task; scientific programmers, who would develop new routine validation algorithms; and system administrators, who would maintain the software.

This document consists of the following main sections:

- Product Overview: provides relevant details of the ABI and a brief description of the products generated by the algorithm.

- Product Requirement Description: provides the detailed requirements for the CMIP key algorithms and software system.

- Algorithm Description: provides the details for CMIP processing outline, input/output parameters and key routine validation algorithms.

- Test Data Sets, and Output: provides a description of the test data sets used to characterize the performance of the algorithms and quality of the data products. It also describes the results of using test data sets.

# 2	PRODUCT OVERVIEW

This section describes the CMIP and the requirements it places on the system.

## 2.1	Products Generated

The CMIP will be produced using all 16 bands in three coverage areas: FD, CONUS, and mesoscale. Bands 1-6 are the visible bands (1 and 2) and near-IR (3-6), while bands 7-16 are IR bands.

The CMIP is responsible for ABI cloud and moisture imagery, and will produce 54 total end-products, including:

- 16 ABI single-band products ([spectral] radiance in bands 1-16) for 3 coverage areas

- Multi-band spectral products (16 bands at 2 km resolution) for 3 coverage areas in NetCDF4 format

- Multi-band spectral products (16 bands at 2 km resolution) for 3 coverage areas in McIDAS format

The three coverage areas are for full disk, CONUS and mesoscale images. Additionally, the CMIP end-products will include the information about the methods of converting radiance to reflectance factor and BV for bands 1-6, and converting radiance to BT and then to BV for bands 7-16. The BV for a pixel on a display is perhaps at lower precision. By community convention the term “radiance” is used throughout this document and actually refers to a spectral radiance; e.g., technically radiance is only at a single frequency, while spectral radiance is radiance per unit frequency (either wavelength or wavenumber). It should also be noted that the radiance is actually an average value, weighted by the sensor response for that given band.

## 2.2	Instrument Characteristics

The ABI on GOES-R has 16 spectral bands. The selection of each covers a variety of environmental applications. ABI Infrared (IR) bands have been chosen to coincide either with spectral absorption features (including those of water vapor bands or CO2 bands) or with regions having little absorption (atmospheric windows) that permit observations of the surface. Visible and near-IR bands have been chosen in order to sense lower tropospheric cloud cover (including fog) and the Earth’s surface (NOAA/NESDIS 2007).

Table 1 summarizes the instrument (nominal) central wavelength, spatial resolution (at the sub-point), and bit-depth characteristics of the ABI data in the data stream transmitted to users—the GOES ReBroadcast (GRB). The instrument has two basic modes of operation:

- Every 15 minutes, ABI will scan the full disk (FD) once, plus the continental United States (CONUS) three times (every 5 minutes). A selectable 1000 km × 1000 km mesoscale area will be scanned every 30 seconds. This has been referred to as mode 3 (or ‘flex’ mode).

- The ABI can be programmed to scan the FD iteratively. The FD image can be acquired in approximately 5 minutes (Schmit et al. 2005). This mode has been referred to as mode 4 or the Continuous Full Disk (CFD).

Table 1. GOES-R ABI instrument characteristics

2.3 Product Requirements

The primary output of this algorithm is the information to generate enhanced cloud and moisture images. These files could also be used to generate the rest of the ABI products. For example, products can access either radiance, reflectance factor or BT. In total, 54 end-products are produced: 16 ABI bands for the 3 coverage areas (CONUS, FD, and mesoscale) in NetCDF format, 1 multi-banded image (all 16 bands) at the spatial resolution of the IR bands (2 km) for each of the 3 coverage areas in NetCDF format, and 1 multi-banded image (all 16 bands) at 2 km resolution for each of the 3 coverage areas in McIDAS format. The supported formats will be in NetCDF (version 4) format and McIDAS format. Output should be able to be converted from NetCDF or McIDAS format to other formats.

# 3	VALIDATION OVERVIEW

This section describes the CMIP software system processing outline, input/output parameters, and key algorithms at their current level of maturity.

## 3.1	Algorithm Overview

The CMIP algorithms consist of acquiring the radiometrically calibrated and mapped radiances and converting to BV. The BV are used as indices either to customized color tables or to the red/green/blue color components of a composite image, resulting in enhanced imagery intending to highlight environmental features of interest. The GOES-R ABI CMIP radiometric algorithms are based on GOES 8-13 algorithms (Weinreb et al. 1997; Schmit et al. 1991). Algorithms for mapping TOA reflectance factor and BT to BV are based on examples from the literature (Acharya and Ray 2005; Russ 2002), commercial remote sensing data visualization tools (e.g., ENVI and PCI), and open source tools (e.g. McIDAS-X/V and IDV). Some display systems may be capable of displaying more that 8 bits (or 256 levels). The bit-depth of the end-products (BV) for CMIP is 12 bits for all the bands, with the exception of band 7, which is 14 bits.  Hence, the enhancements need to be applied on the user display side and not converted to only 8 bits, for example. This is because the conversion to an 8-bit BV also includes a ‘compression’ such as a square-root function in the solar bands and a bi-linear stretch for the IR bands.

The CMIP will convert from the radiances to a number of geophysical parameters, such as brightness temperature or reflectance factor. This is needed in that a number of subsequent products use these units.

# 4	ALGORITHM DESCRIPTION

CMIP algorithms are responsible for:

- Converting the scaled [spectral] radiance from the input data stream (eg, GRB) to a spectral radiance (bands 1-16).

- Re-sampling bands 1, 3, and 5 from 1 km to 2 km for multi-band product. Re-sampling band 2 from 0.5 km to 2 km for multi-band product.

- Converting the spectral radiance to equivalent BT (bands 7-16).

- Calculating BV (bands 1-16).

High-level flowchart for generating CMIP

# Validation Approach

Validation of the Imagery products is slightly different than for the other ABI products.   Imagery products do not all have external data sources to compare to and some validation methods will involve comparing current data to previous data (timeseries analysis).

During the pre-launch phase of the GOES-R program, the product validation activities are aimed at characterizing the performance and uncertainties of the CMIP products resulting from parameterizations and algorithmic implementation artifacts. During this phase, the validation strategies can be tested on current GOES (and other geostationary satellites if possible, with more bands) in addition to the ABI simulated data described in Section 4.1.  Some of the validation error thresholds can be established using similar bands on current instruments.  Post-launch validation will apply lessons learned to inter-comparisons of actual CMIP products generated from real ABI measurements and other observations. Validation methodologies and tools developed and tested during the pre-launch phase will be automated and applied. More specific details on CMIP product validation activities can be found in the Product Validation Document for the CMIP product.  ? Is that the document we’re currently working on?

# 4	GOES-R Simulated ABI Datasets.

4.1

Simulated GOES-R ABI data has been used in the pre-launch phase to develop, test, and validate the ABI CMIP products. The data are derived from use of a radiative transfer model, where the atmospheric and earth surface representations are provided by a high resolution numerical weather prediction forecast model. The GOES-R Algorithm Working Group Proxy Data Team is responsible for the generation of the proxy and simulated instrument data sets. Within the proxy group, the simulated images used by the imagery team were generated by the Cooperative Institute for Meteorological Satellites Studies (CIMSS).

Much work has been done to generate high-quality simulated ABI images based on output from the high resolution Weather Research and Forecasting (WRF) numerical model. The forward radiative transfer models used to transform from model (environmental parameter) space to measurement (radiances and BTs) space cover from the visible, to the IR spectral regions (Otkin and Greenwald 2008). High quality forward model simulations have been made of each of the ABI 16 bands.

The GOES-R AWG Proxy Data team has created several ABI simulations. This section details the work on a CONUS simulation which mimics one of the proposed scan segments on the future ABI (Otkin et. al 2007). Two flexible scanning scenarios are currently under review for the ABI.  The first mode allows the ABI to scan the full disk (FD) every 15 minutes, 3 CONUS scenes, and scan a 1000 km x 1000 km selectable area every 30 seconds, although it is envisioned that 2 areas will be scanned, giving a 1-minute cadence for each. A second mode would program the ABI to scan the FD every 5 minutes (Schmit et al. 2005).

The synthetic GOES-R ABI imagery begins as a high resolution WRF model simulation. The CONUS simulation was performed at the National Center for Supercomputing Applications (NCSA) at the University of Illinois at Urbana-Champaign by the GOES-R AWG proxy data team at CIMSS. Simulated atmospheric fields were generated using version 2.2 of the WRF model (Advance Research WRM (ARW) core). The simulation was initialized at 2355 UTC on 04 June 2005 with 1° Global Forecast System (GFS) data and then run for 30 hours using a triple-nested domain configuration. The outermost domain covers the entire GOES-R viewing area with a 6-km horizontal resolution while the inner domains cover the CONUS and mesoscale regions with 2 km and 0.667 km horizontal resolution, respectively.

WRF model output, including the surface skin temperature, atmospheric temperature, water vapor mixing ratio, and the mixing ratio and effective particle diameters for each hydrometeor species, were ingested into the Successive Order of Interaction (SOI) forward radiative transfer model in order to generate simulated top of atmosphere (TOA) radiances. Gas optical depths were calculated for each ABI infrared band using the Community Radiative Transfer Model (CRTM). Ice cloud absorption and scattering properties were obtained from Baum et al. (2005), whereas the liquid cloud properties were based on Lorenz-Mie calculations.

The WRF-CHEM model is being used to produce near-real-time simulated ABI imagery.  These data differ from the previously mentioned simulations in that they are coarser spatial resolution, only contain CONUS (subset of conus?) domain, and also include aerosols.  The data are produced on S4(?) and though they do not mimic ABI’s temporal frequency, they are frequent enough to begin to test the validation strategies.

CORRELATIVE DATA SOURCE

Testing algorithm and Comparing SEVIRI and the ABI

•Several bands of the ABI were added, in part,  due to SEVIRI plans.

•SEVIRI data are key and being used by many AWG application teams (clouds, aviation, hydrology, trace gas, soundings, cryosphere, land, winds, etc).

•SEVIRI data offers routine 15-min full disk images, similar to the ABI FD data.

•Significant collaborations on the use of SEVIRI data to prepare for GOES-R are on-going.

Kaba, I’m leaving this section alone – I know you want to combine all the “correlative data source’ sections.

I suggest removing the bit on calipso - that’s really for validating cloud algorithms.  We should avoid talking about validating anyone else’s products so we don’t wind up having to do it.

•The SEVIRI imager on MSG offers 11 ABI channels with a temporal resolution of 15 minute – this is the best proxy data available today for our needs.

Combining GOES and MET data (and others)

•The Antarctic composite is a mosaic of GOES, Meteosat, GMS, and NOAA polar orbiting satellite data.

The global composite images combine GOES, Meteosat, and MTSAT satellite data.

# 5                                 Routine Validation Tools

# 5.1 Thumbnail images for routine visualization

All the images shown in Figure 5.1 have already been remapped to the GOES-R ABI Fix Grid Format (FGF). The FGF comprises a set of fixed view angles at regular intervals, and their respective intersections with the GRS80 Earth geoid, from an ideal or nominal point in space in the equatorial plane. The complete description of the FGF is formally defined in ABIPORD13. The transform algorithms are being tested in the McIDAS-V geolocation framework, and necessary CF-compliant metadata have been added to NetCDF files . Using the FGF will reduce the file size (by not needing to carry a latitude/longitude value for each pixel) and should also improve the performance of any remapping functions.

Simulated ABI scaled integer data for these dataset were computed using output from a high-resolution Weather Research and Forecasting model with Chemistry (WRF-CHEM, version 3.0.1) simulation coupled with the NOAA/NESDIS CRTM forward radiative transfer model.

Figure 5.1 Full disk simulated GOES-R ABI bands 01 through 16 of the Pacific at 137°W at 2100 UTC on 26 June 2008, showing reflectance for the visible bands and Brightness temperatures for the IR  bands.

Figure 5.2 CONUS simulated GOES-R ABI bands 01 through 16  at -75°W at for 2100 UTC on 04 June 2005, showing reflectance for the visible bands and Brightness temperatures for the IR  bands.

Figure 5.3 Mesoscale simulated GOES-R ABI bands 01 through 16 at -75°W for 1730 UTC on 19 July 2006, showing reflectance for the visible bands and Brightness temperatures for  the IR  bands.

# 5.2 Routine Image Animation

Visualization and analysis software package such as McIDAS-V or programming

languages like IDL, MATLAB etc. may be used to routinely generate animated

movies. The images  below were generated using the jython scripting capabilities

in McIDAS-V.  These new scripting capabilities in McIDAS-V are currently being

improved to  allow for remotely running on a server.

Figure 5.4  Animations of images are powerful visualization tools that allow analysts to quickly visualize hours or even days of data.

# 5.3 Time series of radiance/brightness temperatures

A routine time series analysis of the individual fields, such as reflectance factor or brightness temperature, for

each band is one way to monitor possible anomalies in the data . They are very useful for inferring image quality.  If there are anomalies in the data, they usually appear as a sudden spike or sudden drop in signal level  instead of a gradual change.

Figure 5.5  GOES-15 Imager mean brightness temperature time series plot from 2012.

Figure 5.5 shows the GOES Imager Water Vapor Mean brightness temperatures over water from 90 S to 90 N and 110 W to 170 W. The x-axis shows time in hours while the y-axis represents average brightness temperatures. The horizontal blue lines highlights places with rapid changes in brightness temperature. Note the big drop in mean temperature late in the day on March 12, and the big increase last in the day on March 16 2012.  Determining when the there is an anomaly and programming a warning will be refined during the pre-launch testing phase.

# 5.4 Statistics of radiance/brightness temperatures

Computing statistics such as those shown below is one way of routinely validating

datasets. Most software such as McIDAS-V have the built-in capability to compute such statistics.  Bad data points in a sample file will usually throw off one or more of the standard statistical tests (Mean, Median, variance, Correlation etc.).

Figure 5.6  Statistical analysis of simulated ABI band 2 in McIDAS-V.

Figure 5.6  compares two different times for GOES-R ABI band02 (0.64um).

In this case, there is strong correlation between the two files (0.98193), with very minimal differences in the mean(0.01082) and very minimal variances within

each individual file (1.669146e+04, 1.723394e+04). Such statistics supports the argument that the differences between the two files are within the expected range for band02 (0.46um) at 10:00UTC.

# 5.5 Images

The three key improvements in GOES-R ABI as compared to the current imager are improvements in resolution (Spectral, Spatial, Temporal).  A good way to verify improved resolution is to generate high quality images at full resolution and zoom into these animated images to look at finer details.

Figure 5.7 Simulated ABI band 01 at full resolution, displayed in McIDAS-V.

By generating high-resolution simulated ABI images at 1minute intervals, we can animate and compare those to current GOES images or even compare images from the same band but different temporal resolutions. Such comparisons can be used to verify image quality and make sure that the output images do meet the expected quality.

Monitor  “forward model calculations” VS  “satellite Observations”

Use ECMWF plot from below here?

Compare to other imagery (e.g Current  GOES, SEVERI, VIIRS)

Comparing current GOES to GOES-R

WRF runs at the Arctic Region Supercomputing Center  Both images shown in ABI projection using McIDAS-V.  September 29, 2011 at 06 UTC.

Figure 33.  Shows comparison between current GOES and simulated GOES-R ABI bands. The images on the left shows current GOES images from the 3.9 µm and the 13.3 µm. The images on the right shows simulated ABI of the

corresponding channels. (i.e. 3.9 µm and 13.3 µm).  Note, of course, that the ABI images are simulated and hence not exactly corresponding to the actual GOES images, although similar features have been resolved

Image Combination

By using the red band and the blue band that already exist in GOES-R ABI and generating  a synthetic green band, we cam can create  “true color” images that shows vegetations, clouds and Ocean when the right color enhancement is applied to it.

“True Color” with “synthetic” green band from ABI simulated data (from CIMSS); image from Don Hillger, RAMMB.

# Band differencing

Unlike current GOES, which has 5 bands, GOES-R ABI has 16 bands.

The ABI bands, in combination, can highlight a range of phenomena. For example, ABI band 2 (0.64 µm) versus 4 (1.378 µm) can delineate high from low clouds during the day, while a normalized difference between band 2 (0.64 µm) and band 3 (0.865 µm) can show vegetation.

Below are some examples showing Simulated ABI band differences displayed in AWIPS with focused range color enhancements.

I don’t see how band differencing helps us validate – it sounds like a new product.

Figure 28. Example of glaciated clouds; simulated ABI band 11-14 (e.g., 8.5 - 11 µm). Note the location of the ice clouds. This difference can be used both day and night (Focused range).

Figure 29. Upper-level information; Simulated ABI Band 14 –8 (11.2 – 6.19 µm). Note the location of potentially over-shooting tops highlighted by the smallest differences (Focused rang

# Data probing  Deep dive?

# Scatter analysis:

Scatter diagrams are a good way to compare two files (different spectral or  different temporal resolutions). It shows the linear relationship between correlation’s coordinates along the X and Y-axis. The built in scatter analysis tool in McIDAS-V has the added capability of selecting and comparing only specific regions within the main domain as well.

# Data Transect:

A data transact is a very efficient way of probing further into a particular region of a dataset and quickly access information on the data distribution within that region. The images shown below were generated using McIDAS-V as an analysis tool to look into the radian distribution of simulated ABI data within a convective cloud for the reflective band 02(0.64um) .

# 5.6 Comparison to other satellites:

Geo-located GOES-15 and MTSAT Mean Water Vapor Brightness temperatures, below, show good agreement until 2045 UTC on March 12, when the SPS change that accessed incorrect values in a different database was implemented. the SPS unit used for the generation of GVAR is noted

When done routinely, this can help us detect calibration as well as navigation issues if any exist.  It is likely that Japan will be operating an ABI-like instrument over the Pacific and an ABI placed as “GOES-West” will be able to compare in nearly all bands.

GOES-15 Imager Calibration Anomaly

(monitored by ECMWF)

Beginning of the anomaly

Encompassing the anomaly

Large observation minus calculation

GOES-15 Imager water vapor band compared to co-located MTSAT values. Note large brightness temperature differences beginning near 21UTC on March 12, 2012. Plus, the SPS unit used for the generation of GVAR is noted (obtained from the GVAR signal).

Image from: CIMSS and STAR.  Should go under calc vs obs section?

# Select Links

## GOES-R:

http://www.goes-r.gov

http://www.goes-r.gov/education/index.html

http://www.meted.ucar.edu/index.htm

http://cimss.ssec.wisc.edu/goes_r/proving-ground.html

## GOES:

http://goespoes.gsfc.nasa.gov/goes/index.html

http://goes.gsfc.nasa.gov/text/goes.databookn.html

http://rapidfire.sci.gsfc.nasa.gov/

http://goes.gsfc.nasa.gov/

http://rammb.cira.colostate.edu/projects/goes-n/

http://rammb.cira.colostate.edu/projects/goes-o/

## UW/SSEC/CIMSS/ASPB:

http://cimss.ssec.wisc.edu/goes_r/awg/proxy/nwp/

http://cimss.ssec.wisc.edu/goes/abi/

http://cimss.ssec.wisc.edu/goes/abi/wf

http://cimss.ssec.wisc.edu/goes/blog/

http://www.ssec.wisc.edu/data/geo/

http://www.ssec.wisc.edu/~jordang/awips-modis/index.html

http://cimss.ssec.wisc.edu/dbs/SatelliteNotes/Notes.html

# Appendix:

McIDAS-V scripting for generating back ground images:

import time

setOffScreen(1)

# command line options have been removed.  Use this script as a template and use sed, awk or whatever works to insert necessary

# parameters.

datadir=""

indir="/Users/kbah/glance/input/calnex/val/"

outdir="/Users/kbah/mcidasv/output/script/calnex/"

aband=["01(0.47um)","02(0.64um)","03(0.86um)","04(1.37um)","05(1.61um)","06(2.25um)","07(3.9oum)","08(6.19m)","09(6.95um)","10(7.34um)","11(8.5um)","12(9.62um)","13(10.35um)","14(11.2um)","15(12.3um)","16(13.3um)"]

ares=["2", "2", "2","2", "2", "2", "2","2","2","2","2","2","2","2", "2", "2"]

times=["0000utc","0100utc","0200utc","0300utc","0400utc","0500utc","0600utc","0700utc","0800utc","0900utc","1000utc","0000utc","1100utc","1200utc","1300utc","1400utc","1500utc","1600utc","1700utc","1800utc","1900utc","2000utc","2100utc","2200utc","2300ut"]

adate=["0501", "0502", "0503", "0504", "0505", "0506", "0507", "0508", "0509", "0510", "0511", "0512", "0513", "0514", "0515", "0516", "0517", "0518","0519", "0520", "0521", "0522", "0523", "0524","0525", "0526", "0527", "0528", "0529", "0530"]

alabel="SIMULATED GOES-R ABI (CalNex) "

b_var=["ABI_Refl_fact_b", "ABI_TBB_b"]

sourceType = 'netcdf.grid'

displayType = 'planviewcolor'

dataMin=None

dataMax=None

cTable="goesr"

projection="CALNEX"

scaleFactor=1.0

width = 800

height = 800

#d=0

for d in range (17, 29):

for i in range(0,1):

for j in range(0, 24):

bb=str(i+1)

ee="_"

visir="ir"

bb=str(i+1).zfill(2)

if i < 6:

ee="_esun_"

visir="vis"

cTable="Square Root Visible Enhancement"

#cTable="Inverse Gray Scale"

pre_name="GRB_VAL"+ee+"band"+bb+"_calnex_chem_rem_2km_sim_abi_fixed_grid_"+visir+"_CRTM_V2.04_2010_"+adate[d]+"_"+times[j]+".nc"

inputFileName1 = indir+pre_name

selectedField1 = b_var[1]+ str(i+1).zfill(2)

outFileName=outdir+ "band"+aband[i]+"_"+ adate[d]+"_"+ times[j]+ ".jpg"

if i < 6:

selectedField1=b_var[0]+str(i+1).zfill(2)

idv.getStateManager().setViewSize(java.awt.Dimension(width,height))        #change the width and height of the display

#### GET THE DATA FOR USE IN McIDAS-V ONLY RUN CODE IF NO ERRORS ARE ENCOUNTERED###

with managedDataSource(inputFileName1, dataType=sourceType) as dd1:

dataSource1 = idv.getDataManager().findDataSource(dd1.getName())

dataChoice1 = dataSource1.findDataChoice(selectedField1)

flatField1 = dataSource1.getData(selectedField1)

#### GET LABELS ####

#### Use Grid Utilities to get color bar label ([0] is selecting the first element in the java array returned from GridUtill.getParamUnits(Choice1))

print type(dd1), type(dataChoice1), selectedField1

colorBarLabel1= GridUtil.getParamUnits(flatField1)[0]

dataDescription1 = dataChoice1.getDescription()

# Display the data and get the displayControl (dc)

dc=createDisplay('planviewcolor',dataChoice1)

time.sleep(5)

# get the view managers for setting lat/lon lines, projections and color tables

myDisplay = firstDisplay()

#set the projection

if type(projection).__name__ != 'NoneType':

myDisplay.setProjectionByName(projection)

#zoom if desired

myDisplay.setScaleFactor(scaleFactor)

#---changing the color table---#

ctm=idv.getColorTableManager()

ct=ctm.getColorTable(cTable)

dc.setColorTable(ct)

# Turn off the layer label

dc.setShowInDisplayList(0)

#### SAVE THE IMAGE ####

writeImage(outFileName,\

"matte background=white top=60;matte background=black bottom=40;\

colorbar width=300 height=10 anchor=UM place=UM,0,20 showlines=true tickmarks=2 showunit=true;\

overlay text="+dataDescription1+"_"+adate[d]+"_"+times[j]+" place=LM,0,-20 anchor=LM color=yellow fontsize=20 fontface=Arial")

#;\

#overlay text="+str(colorBarLabel1)+" anchor=UM place=UM,0,40 color=black fontsize=12 fontface=Arial")

time.sleep(10)

# REFERENCES

Acharya, T., and A. K. Ray> 2005: Image processing: principles and applications: Wiley-Interscience.

Baum, Bryan A., Ping Yang, Andrew J. Heymsfield, Steven Platnick, Michael D. King, Y-X. Hu, Sarah T. Bedka, 2005: Bulk Scattering Properties for the Remote Sensing of Ice Clouds. Part II: Narrowband Models. J. Appl. Meteor., 44, 1896–1911. doi: 10.1175/JAM2309.1

Cao, C., and A. Heidinger. 2002: Proceedings of SPIE (http://www.star.nesdis.noaa.gov/smcd/spb/calibration/icvs/documents/Cao_2002SPIE.pdf).

Clarke, J., 2010: GOES-R ABI LEVEL 1B SOLAR CHANNEL DATA DISTRIBUTION STUDY: Analysis of alternatives for utilization of Radiance and Reflectance Factor Instrument Data.

Michael D.; Hu, Y.-X., and Bedka, S. T., 2005: Bulk scattering properties for the remote sensing of ice clouds, part II: Narrowband models. Journal of Applied Meteorology, Volume 44, Issue 12, pp.1896-1911.

Gurka, J. J., and G. J. Dittberner. 2001: The next generation GOES instruments: Status and potential impact. Paper read at Fifth Symp. on Integrated Observing Systems, Albuquerque, NM.

Hillger, D. W., and T. Schmit. 2004: Quantization noise for GOES-R ABI bands. Paper read at Proceedings of the 13th Conference on Satellite Meteorology and Oceanography, Sep.,  Norfolk, VA.

NOAA/NASA, 2010: GOES-R Ground Segment (GS) Functional and Performance Specification (F&PS): GOES-R/Code 417.

NOAA/NESDIS, 2007: GOES-R Series Mission Requirements Document (MRD, version 3.1): GOES-R Program/Code 417.

Otkin, J. A., D. J. Posselt, E. R. Olson, H.-L. Huang, J. E. Davies, J. Li, and C. S. Velden, 2007:  Mesoscale numerical weather prediction models used in support of infrared hyperspectral measurements simulation and product algorithm development.  J. Atmospheric and Oceanic Tech., 24, 585-601.

Otkin, J. A., and T. J. Greenwald, 2008: Comparison of WRF model-simulated and MODIS-derived cloud data. Mon. Wea. Rev. 136:1957-1970.

Padula, F. and C. Cao, 2011: CWG (Calibration Working Group) Analysis: ABI Max/Min Radiance Characterization and Validation.

Russ, J. C., 2002: The Image Processing Handbook, Fourth Edition CRC Press.

Schmit, T. J., M. M. Gunshor, W. P. Menzel, J. Li, S. Bachmeier, and J. J. Gurka, 2005: Introducing the next-generation Advanced Baseline Imager (ABI) on GOES-R. Bull. Amer. Meteor. Soc. 8:1079-1096.

Schmit, T. J., M. P. Weinreb, and W. P. Menzel, 1991: GVAR IR scale factor coefficients: NOAA.

Weinreb, M. P., M. Jamison, N. Fulton, Y. Chen, J. X. Johnson, J. Bremer, C. Smith, and J. Baucom, 1997. Operational calibration of Geostationary Operational Environmental Satellite-8 and -9 imagers and sounders. Appl. Opt. 36:6895-6904.

Weinreb, M. P., J. X. Johnson, J. H. Lienesch, and D. S. Crosby, 1989: Destriping GOES images by matching empirical distribution functions. Rem. Sens. Environ 29 (185-195).