2012 Lidar

Accuracy Assessment and

Quality Control Report

For the Kewaunee, WI

Project Area

Prepared by:

Groundpoint Technologies, LLC

14 Lund Court

Woodstock, NY 12498

(315) 459-1515

Table of Contents

2.	INTRODUCTION	4

3.	Accuracy Assessment	5

3.1	LiDAR Checkpoint Ground Survey Data	6

3.2	LiDAR Data	8

3.3	Checkpoint Survey/LiDAR Data Post-Processing	8

3.4	Randomly Spaced LiDAR Accuracy Assessment Methodology	9

3.5	Overall Statistics for the Randomly Spaced Lidar Data	10

3.6	ACCURACY ASSESSMENT SUMMARY	13

4.	QUALITY CONTROL REVIEW	14

4.1	Deliverables Review	14

4.1.1	Verify Pre-Flight Deliverables	14

4.1.2	Verify Post-Flight Deliverables	15

4.1.3	Verify Mission Documentation	15

4.2	General Data Review	16

4.3	Technical Data Review	16

4.3.1	Range of Elevations	16

4.3.2	Bare Earth Point Density and Post Spacing	17

4.3.3	Void Areas	18

4.3.4	Artifacts	20

5.	References	21

Appendix A	22

List of Figures

Figure 1 shows the boundary of the Kewaunee, WI 2012 lidar collection area.	4

Figure 2 shows the distribution of surveyed checkpoints by landcover class.	7

Figure 3 shows the distribution of all lidar points in the Kewaunee, WI collection area.	8

Figure 4 shows checkpoints in the Brush, Forest, Urban, and High Grass land cover types displayed with a TIN created from the lidar ground points in the top image.  The TIN colors indicate elevation range.  The bottom image shows the same checkpoints displayed with orthoimagery provided by Pictometry International.	9

Figure 5 is a histogram showing the distribution of error residuals.  A “normal” shaped histogram would be bell shaped and centered on zero.  This histogram is not bell shaped, but within the limits of the FEMA specification for normally distributed lidar data.	11

Figure 6.  Distribution of random tiles selected to test the nominal pulse spacing requirement.	12

Figure 7.  Voids in a field.  No obvious reason for this void area.	19

Figure 8.  Voids in a field near the edge of overlap area.	20

List of Tables

Table 1. Summary of Error Statistics	12

Table 2.  Nominal Pulse Spacing calculation for 14 random tiles in the Kewaunee, WI project area.	13

# INTRODUCTION

LiDAR-based elevation data may be used for hydraulic modeling and floodplain mapping if it meets specific quality assurance and quality control criteria (FEMA 2003, 2010).  GroundPoint Technologies LLC (Groundpoint) performed an independent accuracy assessment and quality control review of the randomly spaced LiDAR data collected in Kewaunee, Wisconsin.  The methodology was designed to meet FEMA guidelines and specifications for flood mapping.  This document presents the results of the accuracy assessment and quality review.

LiDAR data for Kewaunee, WI was collected by Pictometry International Corp (Pictometry) in 5 missions from March 26 – April 20, 2012.  The data was collected at a nominal post spacing of 0.97 meters (3.18 feet) over an area covering approximately 380 square miles (see Figure 1).

Figure 1 shows the boundary of the Kewaunee, WI 2012 lidar collection area.

Pictometry processed the raw point cloud data to ensure proper alignment of the data between flight lines.  The data was then classified for “Ground” by using a series of automated filters in Terra Solid software to remove above ground vegetation and buildings.  Points in water and on bridges were manually removed from the Ground class.  The data was then tiled and saved in the industry standard LAS 1.2 format.  The data was delivered to Groundpoint in classified LAS format in the summer of 2012.

Compared with the 1.19 foot specification for vertical accuracy equivalent to 2-foot

contours in open terrain, tested 0.788 feet fundamental vertical accuracy at 95% confidence level in bare earth terrain. The lidar data collected in the Kewaunee, WI project area meets FEMA vertical accuracy specifications at the “High” specification level with a nominal post spacing less than or equal to 2 meters.  This is equivalent to saying that the data meets the “Highest” FEMA specification level at an equivalent 2 ft contour interval.  The data does not meet FEMA specification for a 1ft equivalent contour interval or the “Highest” specification level with a nominal post spacing less than or equal to 1 meter because it does not meet the non-clustering spacing requirements for being less than or equal to 1 meter NPS.

The accuracy assessment methodology and results are reported in section 2 Accuracy Assessment.

In addition to performing an accuracy assessment, Groundpoint visually inspected the Ground lidar points to assess the completeness and consistency of the dataset, which may not be captured during the accuracy assessment.  Groundpoint also makes sure all required metadata is delivered with the lidar data.  The visual inspection of the data is reported in section 3 Quality Control Review.  The data passed the quality control review.

# Accuracy Assessment

The data must satisfy two conditions to pass the FEMA accuracy assessment specifications.  The data must pass accuracy assessment statistics as defined below and it must meet nominal pulse spacing (NPS) criteria.  The NPS criteria is defined as follows (FEMA, 2010):

Consistent with USGS Lidar Guidelines and Base Specification, v13, a regular grid, with cell size equal to the design NPS*2 will be laid over the first return data within the geometrically usable center portion of each swath. At least 90% of the cells in the grid shall contain at least one lidar point.

The accuracy assessment statistics are calculated by comparing the elevation values of surveyed checkpoint data to the elevation values of the lidar data in different land cover types.  The elevation differences, or error residuals, are used to calculate the Fundamental Vertical Accuracy (FVA), Supplemental Vertical Accuracy (SVA) and Consolidated Vertical Accuracy (CVA) as defined by FEMA below:

Fundamental Vertical Accuracy (FVA) –The FVA is determined with vertical checkpoints located only in open, non-vegetated terrain, where there is a very high probability that the sensor will have detected the ground surface. FVA is calculated at the 95% confidence level in open terrain only, using RMSEz x 1.9600.

Consolidated Vertical Accuracy (CVA) – pertains to all land cover categories combined. Compliance with the CVA specification is mandatory in order for an elevation dataset to qualify for satisfaction of a specified equivalent contour accuracy.   Computed by using the 95th percentile, CVA is always accompanied by Fundamental Vertical Accuracy (FVA).

Supplemental Vertical Accuracy (SVA) – pertains to other major land cover categories representative of the floodplain being mapped. SVA values are target values, where one SVA category can test higher and another lower than the target SVA value so long as the overall CVA is satisfied for the consolidated equivalent contour accuracy.  Computed by using the 95th percentile, SVA is always accompanied by Fundamental Vertical Accuracy (FVA).

The FEMA specifications for vertical accuracy vary depending on the level of flood risk, terrain slope, and type of engineering analysis expected in the project area.  The flood risk in the Kewaunee project area in general is in a moderate to low risk area according to www.floodsmart.gov.  The terrain in the project area is generally flat to rolling.   FEMA states that the highest and high specifications are suitable for all types of engineering analyses.  The Kewaunee lidar data needs to meet FEMA’s High specification level.  See Table 1 below for a summary of accuracy assessment requirements as excerpted from the FEMA Lidar Specifications 2010 document.

Table 1. Vertical Accuracy Requirements based on Flood Risk and Terrain Slope within the Floodplain being

Mapped.

| Level of Flood Risk | Typical Slopes | Specification Level | Vertical Accuracy, 95% Confidence Level FVA/CVA | Lidar Nominal Pulse Spacing (NPS) |
| --- | --- | --- | --- | --- |
| High (Deciles 1,2,3) | Flattest | Highest | 24.5 cm/36.3 cm | ≤1 meter |
| High (Deciles 1,2,3) | Rolling or Hilly | High | 49.0 cm/72.6 cm | ≤2 meters |
| High (Deciles 2,3,4,5) | Hilly | Medium | 98.0 cm/145 cm | ≤3.5 meters |
| Medium (Deciles 3,4,5,6,7) | Flattest | High | 49.0 cm/72.6 cm | ≤2 meters |

# LiDAR Checkpoint Ground Survey Data

A LiDAR checkpoint ground survey was conducted by Red Plains Surveying for the accuracy assessment of the data.  The survey consists of 106 checkpoints that are clustered throughout the project area as seen in figure 2 below.  The checkpoint survey contains points in five different land cover types as follows:

1.	Bare-earth  (e.g., plowed fields, lawns, golf courses), 20 Checkpoints

2.	High Grass, Weeds, and Crops (e.g., overgrown fields and hay) 21 Checkpoints

3.	Brush (e.g., scrubby low trees and bushes), 22 Checkpoints

4.	Forested (e.g., trees), 20 Checkpoints

5.	Urban areas (e.g., impervious surfaces, high, dense manmade structures), 23 Checkpoints

Figure 2 shows the distribution of surveyed checkpoints by landcover class.

FEMA recommends 20 survey points per class. All classes have 20 or more checkpoints.  Digital photos and field sketches are also required for each checkpoint and the surrounding area to verify land cover and other conditions.  All of the points include field sketches and photos.

The LiDAR Checkpoint Survey Data for the collection area is available in Appendix A.  The Easting and Northing (X and Y) points are in NAD83, State Plane Coordinate Zone Wisconson Central, US feet, and the elevation (Z) is in NAVD 88, Geoid 2009, US feet.  The coordinate system of the survey points corresponds to the coordinate system of the lidar data.

# LiDAR Data

The LiDAR data was delivered by Pictometry via ftp.  There are 381 LiDAR tiles, all in LAS 1.2 format.  The LAS files contain all points classified as either “Unclassified”, “Ground”, or “Low Point/Noise”.  The data was delivered in NAD83, State Plane Wisconsin Central horizontal coordinate reference system and NAVD88 vertical datum adjusted to GEOID ’09 (all units in US feet).

# Checkpoint Survey/LiDAR Data Post-Processing

The check point survey data was delivered in excel format.  The excel file was converted to a shapefile and brought into an ESRI ArcMap project along with the LiDAR collection area index.  The checkpoints were well distributed throughout the region (See Figure 3 below).  Groundpoint used LP360 software, a plugin for ArcGIS, to perform the accuracy assessment.

Figure 3 shows the distribution of all lidar points in the Kewaunee, WI collection area.

# Randomly Spaced LiDAR Accuracy Assessment Methodology

In order to compare the elevation values of the checkpoints to the LiDAR data a TIN surface was created from the LiDAR points and a point shapefile was created from the XYZ values of the table of checkpoints. A TIN is a 3D surface that preserves the precision of the LiDAR data points while simultaneously modeling the values between the points.  Figure 4 shows an example of the TIN created for performing the accuracy assessment from the check point survey.

Figure 4 shows checkpoints in the Brush, Forest, Urban, and High Grass land cover types displayed with a TIN created from the lidar ground points in the top image.  The TIN colors indicate elevation range.  The bottom image shows the same checkpoints displayed with orthoimagery provided by Pictometry International.

LP360 software tools and Microsoft Excel were used to implement a randomly spaced LiDAR accuracy assessment.  LP360 was used to compute spot elevations from the TIN surface at the location of each survey point.  The results were exported to a text file and brought into Microsoft Excel for the statistical analysis.

# Overall Statistics for the Randomly Spaced Lidar Data

FEMA defines vertical accuracy requirements based on flood risk and terrain slope within the floodplain.  For example at the highest level of engineering analysis for risk mapping, in the flattest terrain and at the highest level of flood risk FEMA specifies the vertical accuracy of the data must have a Fundamental Vertical Accuracy (FVA) no greater than 24.5 centimeters (9.65 inches) at the 95% confidence level and a lidar nominal post spacing no greater than 1 meter. These specifications are equivalent to the specifications required for 1 foot contour mapping.

The Kewaunee, WI level of flood risk is generally in the low to moderate risk category.  The terrain is generally flat to rolling.  The engineering analysis is expected to fall in the High range for medium to high flood risk, flat to rolling terrain and would be appropriate for 2 foot contour mapping with a nominal post spacing no greater than 2 meters.  Therefore the vertical accuracy requirements of the Kewaunee lidar data require an FVA at the 95% confidence level to be no greater than 49 cm, the consolidated vertical accuracy no greater than 72.6 cm at the 95th percentile and the lidar nominal post spacing no greater than 2 meters.

FVA is calculated for lidar points in open ground only by multiplying the root-mean-square error (RMSE) by 1.9600. RMSE is the square root of the average of the squared elevation differences between data set elevation values and checkpoint elevation values for identical points.

RMSE = CheckpointZ – LidarZ2n

The data is checked for a normal distribution by using a frequency histogram and measures of kurtosis and skew.  Kurtosis describes the curve of the distribution; whether it is more flat or peaked than normal.  The skew gives an indication of whether there are systematic errors or biases in the data, which would make the curve lopsided.  Data with a normal distribution has a bell shape on the frequency histogram with a zero mean, zero kurtosis, and zero skew.  LiDAR data rarely meets these three criteria exactly for a normal distribution; therefore FEMA specifies limits for Kurtosis and Skew.

The open ground lidar data (Ground and Urban classes) meets the FEMA specifications for a normal distribution (See Figure 5).  The mean elevation difference is 0.432 ft (5.184 in) meaning the lidar data elevation values are on average below true ground elevation values.

Figure 5 is a histogram showing the distribution of error residuals.  A “normal” shaped histogram would be bell shaped and centered on zero.  This histogram is not bell shaped, but within the limits of the FEMA specification for normally distributed lidar data.

The shape of the curve is a little more peaked than normal with a Kurtosis of 0.647.  The error distribution is skewed slightly to the left of mean with a Skew of -0.096.  FEMA recommends any skew value exceeding the absolute value of 0.5 be investigated to determine whether there is a valid reason why the errors do not have a normal distribution since the RMSE calculation is only valid on data with a normal distribution.  This data meets the FEMA criteria for a normal distribution.

The FVA at the 95% confidence level for the open ground lidar data is calculated by multiplying RMSE by 1.960.  The FEMA specification requires the FVA to be no greater than 1.608 ft at the High level of engineering analysis.  This data meets the FEMA specifications with an FVA of 0.788 ft.

The CVA for a combination of all the non-open ground land cover categories and SVA for each land cover category are calculated using the 95th Percentile statistic on an array of the error residuals.  This statistic is different than the FVA statistic and is used because lidar data in land cover categories other than open ground rarely follow a normal distribution, which render an RMSE statistic invalid.  The FEMA specification at the High level of engineering analysis for CVA and SVA is a maximum of 1.608 ft. All CVA and SVA statistics for this lidar data set meet the FEMA specifications.  The statistical analysis for the lidar data are shown in table 1.

Table 1. Summary of Error Statistics

| Land Cover Type | # of Checkpoints Used in Analysis | Mean Error (ft) | Standard Deviation (ft) | Minimum Elevation Difference Value (ft) | Maximum Elevation Difference Value (ft) | RMSEz (ft) | FVA (RMSEz * 1.960) spec = 1.608 ft | CVA (95th Percentile) spec = 2.382 ft | SVA (95th Percentile) spec = 2.382 ft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FVA (BareEarth) | 20 | 0.372 | 0.157 | 0.059 | 0.722 | 0.402 | 0.788 |  | 0.715 |
| CVA |  |  |  |  |  |  |  | 0.651 |  |
| SVA |  |  |  |  |  |  |  |  |  |
| Brush | 22 | 0.149 | 0.347 | -0.763 | 0.540 | 0.370* |  |  | 0.532 |
| Forested | 20 | 0.232 | 0.254 | -0.301 | 0.655 | 0.339 |  |  | 0.646 |
| High Grass | 21 | 0.252 | 0.281 | -0.748 | 0.545 | 0.373* |  |  | 0.540 |
| Urban | 23 | 0.485 | 0.140 | 0.259 | 0.774 | 0.504 |  |  | 0.763 |

* Exceeds skew limits for a normal distribution of error.

The survey checkpoints are considered to be “true ground”.  When the resulting error residual is positive the lidar data is below true ground.  When the resulting error residual is negative then the lidar data is above true ground.  Table 2 contains the Z value of all the points by landcover type, the checkpoint Z value and the error residual (CheckpointZ - LidarZ).  All error residuals and statistical analysis is located in Appendix A.

In addition to the error statistics listed above, FEMA also requires a minimal nominal pulse spacing (NPS) be met to satisfy specification levels.  The NPS is determined by using a regular grid, with cell size equal to the design NPS*2, which is equivalent to 2 meters for the Kewaunee project area.  The regular grid is laid over the first return data within the geometrically usable center portion of each swath. At least 90% of the cells in the grid must contain at least one lidar point.

Figure 6.  Distribution of random tiles selected to test the nominal pulse spacing requirement.

Fourteen random lidar tiles were used as a sample to determine if the NPS for the Kewaunee project area meets the above specification.  See distribution of the tiles in figure 6.  ESRI conversion tools were used to convert the first return lidar point data to a raster with a 2m cell size.  Any cells that did not have at least one point during the conversion process are assigned a NoData cell value.  The raster was reclassified as follows:

Cells with any value:  reclassified to 1

NoData Cells:  reclassified to 0

The attribute table of the reclassified raster was used to calculate the percentage of raster cells that contained at least one data value.  Table 2 shows the results for each tile.  On average, 99.6% of the cells are filled with at least one first return lidar point at a 2 meter grid cell resolution.

Table 2.  Nominal Pulse Spacing calculation for 14 random tiles in the Kewaunee, WI project area.

| TileID | Total Cells | Value = 1 | Percent Filled |
| --- | --- | --- | --- |
| 30 | 659344 | 659034 | 99.95 |
| 37 | 659344 | 657884 | 99.78 |
| 44 | 659344 | 652007 | 98.89 |
| 90 | 659344 | 656782 | 99.61 |
| 114 | 659344 | 654642 | 99.29 |
| 121 | 659344 | 658684 | 99.90 |
| 192 | 659344 | 658265 | 99.84 |
| 200 | 659344 | 657288 | 99.69 |
| 240 | 659344 | 654589 | 99.28 |
| 291 | 659344 | 657583 | 99.73 |
| 314 | 659344 | 659183 | 99.98 |
| 315 | 659344 | 659102 | 99.96 |
| 350 | 659344 | 656960 | 99.64 |
| 358 | 659344 | 651129 | 98.75 |

# ACCURACY ASSESSMENT SUMMARY

The Kewaunee, WI lidar data meets the FEMA specifications for a High engineering analysis. The open ground (Bare Earth) land cover types meets the specifications for a normal distribution.  The FVA of 0.788 ft (24.0182 cm) also meets the FEMA requirement to be less than 49.0 cm in the High specification level.  The RMSE in open ground is 0.459 ft (13.990 cm), which also meets the FEMA specification for 2 foot contour mapping.  The data meets the 2 meter NPS requirements in the High engineering analysis as well.  Individually, all of the ground cover categories also meet this standard.  The excel spreadsheet containing all analysis is available in Appendix B.

# QUALITY CONTROL REVIEW

The methodology Groundpoint presents here for reviewing the LiDAR quality control implements the FEMA and USGS guidelines (FEMA, 2003, 2010 and USGS, 2009). Groundpoint coordinated with Pictometry to obtain all available information.

# Deliverables Review

# Verify Pre-Flight Deliverables

Pictometry provided pre-flight as summarized in the FEMA checklist below:

| Pre-flight Review Checklist | Pass/Fail | Comments |
| --- | --- | --- |
| Planned lines – sufficient coverage, spacing, and length | Pass | WIKEWA_FlightLines.shp |
| Planned GPS stations | Pass | MIMN, MIOG, MITC, SUP2, WIM5 |
| Planned ground control – sufficient to control and boresight | Pass | GCP folder |
| Calibration plans | Pass | Flightplan folder |
| Vendor quality procedures | Pass | No collection in precip, on the fly Swath check |
| Lidar sensor scan set – planned for proper scan angle, sidelap, design pulse. | Pass | WIKEWA Flightplan.jpg |
| Aircraft utilizes ABGPS | Pass |  |
| Sensor supports project design pulse density | Pass |  |
| Type of aircraft – supports project design parameters | Pass | Piper PA-23 AZTEC |
| Reflight procedure – tracking, documenting, processing | Pass | CHECK SWATH DATA IN FIELD AND SEND SWATHS TO PICTOMETRY FOR REVIEW |
| Project design supports accuracy requirements of project | Pass |  |
| Project design accounts for land cover and terrain types | Pass | CREW CONSTANTLY MONITORING WEATHER, DEM IMPORTED INTO FLIGHTPLAN TO CHECK FOR ELEVATION CONCERNS |

# Verify Post-Flight Deliverables

Pictometry provided post-flight deliverables as summarized in the FEMA checklist below.

| Checklist for QA of Flight Logs | Included Yes/No | Comments |
| --- | --- | --- |
| Flight logs – job #/name | Yes | Daily Reports Folder |
| Flight logs – block or AOI | Yes |  |
| Flight logs – date | Yes |  |
| Flight logs – aircraft tail # | Yes |  |
| Flight logs – lines - # | Yes |  |
| Flight logs – lines - direction | Yes |  |
| Flight logs – lines – start/stop | Yes |  |
| Flight logs – lines – altitude | Yes |  |
| Flight logs – lines – scan angle | Yes |  |
| Flight logs – lines – speed | Yes |  |
| Flight logs – conditions | Yes |  |
| Flight logs – comments | Yes |  |
| Flight logs - pilot name | Yes |  |
| Flight logs - operator name | Yes |  |
| Flight logs - AGC switch | No |  |
| Flight logs – GPS base stations | No | WIKewa Bases.xls, WIKEWA_Bases_WIC_NAD83_ift.shp |

# Verify Mission Documentation

Pictometry provided mission documentation deliverables as summarized in the FEMA checklist below.

| Checklist for Aerial Acquisition Report | Included? Yes/No | Comments |
| --- | --- | --- |
| GPS base station – names | Yes | WIKewa Bases.xls |
| GPS base station – lat/longs | Yes | WIKewa Bases.xls |
| GPS base station – heights | Yes | WIKewa Bases.xls |
| GPS base station – map | Yes | WIKEWA bases KMZ |
| GPS quality – separation plot | Yes | * GPS Separation.pdf |
| GPS quality – PDOP plot | Yes | * PDOP.pdf |
| GPS quality - horizontal Acc. | Yes | *_RMS Error.pdf |
| GPS quality - vertical Acc. | Yes | *_RMS Error.pdf |
| Sensor calibration process | No | Calibration Lines shapefile included. Sensor calibration process conveyed via email. Sensor is lab and field calibrated by Optech. The data is calibrated using the calibration cross lines and TerraSolid application Terramatch to adjust for Heading, Roll, Pitch, Mirror scale, and Z value. |
| Verification of AOI coverage | Yes | WIKEWA Coverage WIC NAD83 sft.shp |
| As-flown trajectories | Yes | Flight Line Trajectories folder and WIKEWA_Trajectories.shp |
| Ground control layout | Yes | WIKEWA_Bases_WIC_NAD83_ift.shp |
| Data verification process documented | Yes | Opened each individual tile and made manual corrections where necessary. |

# General Data Review

Pictometry delivered the LiDAR data in LAS 1.2 format to in the summer of 2012.  Groundpoint performed the QAQC on the ground points and first return points of the data set.  Groundpoint used QCoherent’s LP360 and ESRI’s ArcGIS software for all data processing.

| Checklist for QA of Terrain Products | Pass/Fail | Comments |
| --- | --- | --- |
| Vertical datum correct | Pass | NAVD88, GEOID 2009 |
| Horizontal datum correct | Pass | NAD83 |
| Projection correct | Pass | State Plane Wisconsin Central |
| Vertical units correct | Pass | US foot |
| Horizontal units correct | Pass | US foot |
| Each return contains – GPS week, GPS second, easting, northing, elevation, intensity, return # and classification | Pass |  |
| No duplicate entries | Pass | None found |
| GPS second reported to nearest microsecond | Pass |  |
| Easting, northing, and elevation reported to nearest 0.01 m or 0.01 ft | Pass |  |
| Classifications correct – 1. Unclassified; 2. Bare-earth ground; 7. Noise; 9. Water; 10. Ignored ground; 11. Withheld | Pass | The classified LAS files do not contain the Ignored or Withheld classes. These points are available as a separate set of LAS files that contain all of the unclassified points. |
| Cloud file structure conforms to project tile layout | Unknown |  |
| Naming conforms project requirements | Unknown |  |
| Deliverable tiles checked for significant gaps not covered by aerial acquisition checks and/or caused by data post-processing/filtering | Pass |  |

# Technical Data Review

# Range of Elevations

The minimum and maximum elevation locations were extracted from the las tiles.  The minimum elevation of 576.72 ft is located along Lake Michigan and is the elevation assigned to the waterbody breakline along the shoreline.  The maximum elevation of 1022.19 ft is located in the west in tile 278.

Elevation Range

Min:  576.72 ft

Max:  1022.19 ft

# Bare Earth Point Density and Post Spacing

The 381 tiles in Kewaunee, WI contained a total of 714,811,527 ground points (out of 1,087,811,081 total classified points), and covered an area of 380 mi2.  This equates to an average of 0.067 ground points per ft2, and taking the square root of the reciprocal yields an average ground post spacing of 3.85 ft.

Point Spacing

Point Density = 0.067 ground points per ft2

Bare Earth Post Spacing = 3.85 ft

The spatial distribution of the points is also expected to be uniform and free from clustering.  The USGS provides methodology to check for uniform density:

- A regular grid, with cell size equal to the design NPS will be laid over the data.

- At least 90% of the cells in the grid shall contain at least 1 lidar point.

- Clustering will be tested against the 1st return only data.

- Acceptable data voids identified previously in this specification are excluded.

To test the spatial distribution requirement, the 14 sample tiles that were used to test for the 2m NPS requirements in the accuracy assessment were used again for this test.  The first return points were converted to rasters at the design NPS, which is ~ 3ft (0.97 meters), and 4ft, which is closer to the point spacing of 3.85 listed above.  Any raster cells that did not contain at least one point during the conversion process results in cells that have NoData as the cell value.  The rasters were reclassified as follows:

Cells with any value:  reclassified to 1

NoData Cells:  reclassified to 0

The attribute table of the reclassified raster was used to calculate the percentage of raster cells that contained at least one data value.  The average percentage of cells filled with at least one point at the 3ft grid spacing is 65%.  The average percentage of cells filled with at least one point at the 4ft grid spacing is 91%.  Therefore it is reasonable to conclude that the data is free from clustering around a 4ft NPS but not free from clustering as you get closer to the design NPS of 3.2ft.

# Void Areas

Data voids are areas with no lidar points.  Data voids are measured using the USGS (2009) specifications as follows:

areas => (4*NPS)2, measured using 1st-returns only  within a single swath are not acceptable, except:

- where caused by water bodies

- where caused by areas of low near infra-red (NIR) reflectivity such as asphalt or composition roofing.

- where appropriately filled-in by another swath.

Data voids are not acceptable with LiDAR system malfunction or flight error.  Data voids need to be flagged in areas where LiDAR points have been removed due to dense vegetation.  If the data voids in areas of dense vegetation are less than 1 acre then the voids may usually be filled by interpolation.  If the data voids are greater than 1 acre then cross sections must be cut to fill the void areas.

To create the void areas, the first return points were converted to rasters using ESRI’s “Feature to Raster” tool with an output cell size of 12 ft squared (144 square feet).  The rasters were reclassified so that raster cells with data values were reclassified to “NoData” and raster cells that did not have any values were reclassified to “1”.  The raster was then converted to a polygon.  A field was added for landcover.

The void feature class was layered over the orthoimagery provided by Pictometry in order to determine the landcover in each void area (e.g., water, buildings, vegetation), and therefore whether the void was acceptable or not.  This information was recorded for each void area in the Landcover field in the attribute table.

A total of 9950 data voids were identified.  Of these 9643 are in water or wetland or wet field areas.  There are 304 voids that are 144 square feet in size and are located in fields.  All of these voids are on the edge of a flightline, where two flights come together in an overlap area.  The nominal pulse spacing becomes a little more clustered in these areas leaving little gaps in coverage.  There are 3 voids that are a little larger, 288 square feet and 5904 square feet that are also located in a field.  There is no apparent reason for these voids according to the imagery.  It does not look like lidar system malfunction.  See images below for examples of voids discussed above.

Figure 7.  Voids in a field.  No obvious reason for this void area.

Figure 8.  Voids in a field near the edge of overlap area.

# Artifacts

Visual inspection of the LiDAR data was done using a hillshade derived from a countywide, 5 foot resolution DEM of the LiDAR ground points as well as a TIN generated on the fly using LP360 software.  The hillshade was initially inspected at 1:5000 scale.  Any anomalies (artifacts) in the data were investigated using the orthoimagery provided by Pictometry and/or “bird’s eye” oblique imagery from www.bing.com\maps , and the original all points lidar data.  Artifacts were flagged if the change in elevation from the surrounding ground elevation was greater than +/- 3 feet.

The following categories of artifacts were checked.  No artifacts were found in the Kewaunee dataset.

Excessive noise:  not present

Elevation steps:  none present

Other anomalies:  none found

Correct classification such as bridges, buildings, or trees left in the Ground class:  no artifacts found, however the overlap points are removed from the classified las tiles instead of being classified as Withheld in class 11.

# Quality Control Summary

The data meets FEMA quality control specifications for data that would be used at the High level of engineering analysis.  The NPS at 3.85ft is slightly larger than the design NPS of 3.18ft, but is still well within the engineering specifications of a 2m NPS.  There are a couple of unacceptable data voids, but they are so small that they could easily be interpolated across.  The only artifact type issue is with the classification of Overlap points.  The Overlap points are available in the full, unclassified point cloud, therefore this is not deemed to be an issue either.

# References

- Bellamo, Doug A. Memo to Regional Risk Analysis Branch Chiefs, Procedure Memorandum No. 61 – Standards for Lidar and Other High Quality Digital Topography, FEMA, 27 September 2010.

- Geospatial Positioning Accuracy Standards, Part 3: National Standard for Spatial Data Accuracy, Federal Geographic Data Committee (FGDC), 1998, from http://www.fgdc.gov/standards/projects/FGDC-standards-projects/accuracy/part3/chapter3.

- Map Modernization Guidelines and Specifications for Flood Hazard Mapping Partners, Appendix A: Guidance for Aerial Mapping and Surveying, Federal Emergency Management Agency (FEMA), April 2003, from http://www.fema.gov/plan/prevent/fhm/dl_cgs.shtm

Guidelines for Digital Elevation Data, version 1.0, National Digital Elevation

Program (NDEP), May 10, 2004.

ASPRS Guidelines: Vertical Accuracy Reporting for Lidar Data, version 1.0,

May 24, 2004.

US Geological Survey Program Announcement 10HQPA0014, Base Lidar Specification for the projects funded under the American Recovery and Reinvestment Act of 2009, Attachment A, 01 October 2009.

## Appendix A

The Lidar Checkpoint Survey Data

| Point # | Land cover type | Northing | Easting | Elevation |
| --- | --- | --- | --- | --- |
| 100 | 5 | 313336.238 | 2517720.063 | 822.094 |
| 101 | 2 | 314135.486 | 2528139.022 | 761.793 |
| 102 | 5 | 314849.722 | 2548602.411 | 679.874 |
| 103 | 1 | 315046.146 | 2565181.824 | 675.215 |
| 104 | 1 | 316822.537 | 2576903.284 | 702.828 |
| 105 | 3 | 324876.382 | 2577481.749 | 682.248 |
| 106 | 5 | 324870.292 | 2563130.423 | 668.126 |
| 107 | 4 | 321391.016 | 2547557.300 | 740.620 |
| 108 | 2 | 324047.820 | 2533517.702 | 728.027 |
| 109 | 3 | 324396.729 | 2526251.783 | 779.181 |
| 110 | 4 | 322319.610 | 2569989.677 | 670.209 |
| 118 | 1 | 348433.326 | 2518752.284 | 797.271 |
| 119 | 5 | 345898.998 | 2522050.614 | 775.918 |
| 120 | 2 | 350955.343 | 2529045.699 | 740.997 |
| 121 | 3 | 335694.214 | 2519233.063 | 785.572 |
| 122 | 4 | 337675.844 | 2531978.432 | 767.240 |
| 123 | 3 | 335118.563 | 2538006.473 | 689.231 |
| 124 | 2 | 335140.920 | 2544029.915 | 732.702 |
| 125 | 4 | 335184.555 | 2551532.628 | 715.013 |
| 126 | 1 | 335768.805 | 2555400.310 | 718.773 |
| 127 | 2 | 335685.537 | 2566952.760 | 691.788 |
| 128 | 5 | 335361.014 | 2572046.379 | 679.433 |
| 129 | 1 | 347303.471 | 2537180.180 | 721.531 |
| 130 | 3 | 344140.221 | 2546458.527 | 759.016 |
| 131 | 2 | 346616.335 | 2554567.719 | 735.508 |
| 132 | 4 | 345905.887 | 2561143.036 | 732.720 |
| 133 | 5 | 345789.110 | 2567901.480 | 717.497 |
| 134 | 3 | 350090.874 | 2575923.818 | 687.211 |
| 135 | 2 | 359809.434 | 2575378.913 | 651.684 |
| 136 | 5 | 355400.541 | 2571173.095 | 700.447 |
| 137 | 2 | 356775.437 | 2560688.082 | 669.936 |
| Point # | Land cover type | Northing | Easting | Elevation |
| 138 | 5 | 355847.517 | 2548798.703 | 741.839 |
| 139 | 1 | 356107.504 | 2541792.469 | 765.984 |
| 140 | 2 | 357601.192 | 2527741.686 | 754.133 |
| 141 | 4 | 354160.584 | 2518819.268 | 826.284 |
| 142 | 1 | 361800.215 | 2531871.420 | 784.056 |
| 143 | 4 | 361618.294 | 2552535.684 | 657.703 |
| 144 | 3 | 362734.798 | 2570242.364 | 642.325 |
| 145 | 2 | 366022.703 | 2519354.623 | 730.938 |
| 146 | 5 | 366262.106 | 2527437.264 | 819.130 |
| 147 | 5 | 366574.656 | 2538223.092 | 732.768 |
| 148 | 1 | 368897.450 | 2560842.718 | 615.434 |
| 149 | 4 | 365050.311 | 2571671.746 | 627.682 |
| 150 | 3 | 362714.448 | 2545066.232 | 709.930 |
| 151 | 4 | 375699.392 | 2516987.799 | 758.203 |
| 152 | 4 | 377155.545 | 2548255.976 | 659.512 |
| 153 | 2 | 379819.720 | 2559499.266 | 598.696 |
| 154 | 1 | 374620.461 | 2537153.756 | 689.414 |
| 155 | 5 | 376807.178 | 2527349.040 | 765.041 |
| 156 | 5 | 380480.803 | 2530905.384 | 708.497 |
| 157 | 5 | 380739.902 | 2544696.742 | 597.307 |
| 158 | 4 | 387953.756 | 2538782.976 | 700.039 |
| 159 | 2 | 387396.348 | 2526775.996 | 757.803 |
| 160 | 3 | 386520.565 | 2520451.501 | 721.193 |
| 161 | 2 | 390575.339 | 2550993.247 | 680.426 |
| 162 | 4 | 397307.580 | 2526904.834 | 717.446 |
| 163 | 5 | 397807.395 | 2517161.459 | 767.922 |
| 164 | 1 | 385063.975 | 2546198.386 | 683.564 |
| 165 | 1 | 387176.218 | 2562505.247 | 587.729 |
| 170 | 4 | 396931.442 | 2568737.082 | 670.641 |
| 171 | 4 | 397158.345 | 2561838.000 | 675.040 |
| 172 | 1 | 397618.214 | 2543397.076 | 701.290 |
| 173 | 1 | 400991.697 | 2551731.513 | 703.863 |
| 174 | 1 | 398958.924 | 2535859.050 | 756.369 |
| 175 | 5 | 402609.767 | 2527282.851 | 746.625 |
| 176 | 5 | 405255.908 | 2555676.824 | 668.133 |
| 177 | 2 | 406090.648 | 2537895.751 | 671.328 |
| 178 | 4 | 407989.730 | 2519887.034 | 708.640 |
| 179 | 2 | 414006.454 | 2523155.038 | 716.745 |
| Point # | Land cover type | Northing | Easting | Elevation |
| 180 | 2 | 408085.792 | 2538172.757 | 662.567 |
| 181 | 3 | 407865.319 | 2551844.913 | 649.371 |
| 182 | 3 | 407175.819 | 2558167.412 | 644.932 |
| 183 | 1 | 410800.007 | 2564049.035 | 656.870 |
| 189 | 4 | 422588.821 | 2560015.400 | 706.116 |
| 190 | 4 | 422907.393 | 2560103.330 | 705.677 |
| 191 | 5 | 415162.626 | 2558596.500 | 648.253 |
| 192 | 1 | 419035.252 | 2558752.128 | 691.083 |
| 193 | 1 | 420066.038 | 2549006.011 | 654.598 |
| 194 | 2 | 419792.082 | 2544616.217 | 694.783 |
| 195 | 3 | 419907.654 | 2536933.233 | 704.144 |
| 196 | 2 | 418600.775 | 2529806.075 | 737.726 |
| 197 | 2 | 419497.967 | 2520270.590 | 782.428 |
| 198 | 4 | 419287.348 | 2517998.953 | 757.500 |
| 199 | 4 | 430613.093 | 2553885.037 | 659.464 |
| 200 | 3 | 434219.129 | 2548616.919 | 651.937 |
| 201 | 5 | 427814.721 | 2545793.966 | 700.725 |
| 202 | 5 | 430334.012 | 2541534.458 | 689.828 |
| 203 | 1 | 430584.433 | 2537984.572 | 692.402 |
| 204 | 2 | 432969.523 | 2526923.117 | 750.235 |
| 205 | 3 | 429067.995 | 2524874.211 | 733.484 |
| 206 | 4 | 430057.393 | 2517550.938 | 743.911 |
| 207 | 3 | 435103.733 | 2563884.982 | 679.358 |
| 208 | 1 | 432877.039 | 2564166.941 | 685.571 |
| 209 | 1 | 438033.365 | 2559474.853 | 662.188 |
| 210 | 1 | 438682.236 | 2550585.239 | 689.820 |
| 211 | 5 | 439545.221 | 2543013.115 | 654.220 |
| 212 | 3 | 440768.513 | 2532524.281 | 684.631 |
| 213 | 4 | 438942.931 | 2527051.496 | 779.074 |
| 214 | 2 | 439721.139 | 2525690.659 | 753.887 |
| 215 | 2 | 440426.271 | 2517526.447 | 769.169 |